CABOS, CONECTORES E CONEXÕES.
	 
	 

	 
	  [image: image1.png]


[image: image2.png]





	 
	ATX Power Supply
	Mouse (PS/2)

	 
	Keyboard (5 PC)
	Keyboard (MINI-DIN)

	 
	Serial (PC 9)
	Serial (PC 25)

	 
	Parallel (PC)
	ATA Internal

	 
	RJ-45
	S-Video

	 
	Sony Playstation Port
	TABELA AWG

	 
	 
	 

	 
	ATX Power Supply
	 

	 
	[image: image3.png]



	20 PIN MOLEX 39-29-9202 at the motherboard

	 
	[image: image4.png]



	20 PIN MOLEX 39-01-2200 at the cable


	Pin
	Name
	 
	Color
	Description

	1
	3.3V
	 
	Orange
	+3.3 VDC

	2
	3.3V
	 
	Orange
	+3.3 VDC

	3
	COM
	 
	Black
	Ground

	4
	5V
	 
	Red
	+5 VDC

	5
	COM
	 
	Black
	Ground

	6
	5V
	 
	Red
	+5 VDC

	7
	COM
	 
	Black
	Ground

	8
	PWR_OK
	 
	Gray
	Power Ok (+5V & +3.3V is ok)

	9
	5VSB
	 
	Purple
	+5 VDC Standby Voltage (max 10mA)

	10
	12V
	 
	Yellow
	+12 VDC

	11
	3.3V
	 
	Orange
	+3.3 VDC

	12
	-12V
	 
	Blue
	-12 VDC

	13
	COM
	 
	Black
	Ground

	14
	/PS_ON
	 
	Green
	Power Supply On (active low)

	15
	COM
	 
	Black
	Ground

	16
	COM
	 
	Black
	Ground

	17
	COM
	 
	Black
	Ground

	18
	-5V
	 
	White
	-5 VDC

	19
	5V
	 
	Red
	+5 VDC

	20
	5V
	 
	Red
	+5 VDC


	 
	 
	 

	 
	Mouse (PS/2)
	



	 
	[image: image6.png]



	6 PIN MINI-DIN FEMALE (PS/2 STYLE) at the computer.


	Pin
	Name
	Description

	1
	DATA
	Key Data

	2
	NC
	Not connected

	3
	GND
	Ground

	4
	VCC
	Power , +5 VDC

	5
	CLK
	Clock

	6
	NC
	Not connected


	 
	 
	 

	 
	Keyboard (5 PC)
	



	 
	[image: image8.png]



	5 PIN DIN 180° (DIN41524) FEMALE at the computer.


	Pin
	Name
	Description
	Technical

	1
	CLOCK
	Clock
	CLK/CTS, Open-collector

	2
	DATA
	Data
	RxD/TxD/RTS, Open-collector

	3
	n/c
	Not connected
	Reset on some very old keyboards.

	4
	GND
	Ground
	 

	5
	VCC
	+5 VDC
	 


	 
	 
	 

	 
	Keyboard (MINI-DIN)
	



	 
	[image: image10.png]



	6 PIN MINI-DIN FEMALE (PS/2 STYLE) at the computer.


	Pin
	Name
	Description

	1
	DATA
	Key Data

	2
	n/c
	Not connected

	3
	GND
	Ground

	4
	VCC
	Power , +5 VDC

	5
	CLK
	Clock

	6
	n/c
	Not connected


	 
	 
	 

	 
	Serial (PC 9)
	



	 
	[image: image12.png]



	9 PIN D-SUB MALE at the Computer.


	Pin
	Name
	RS232
	V.24
	Description

	1
	CD
	CF
	109
	Carrier Detect

	2
	RXD
	BB
	104
	Receive Data

	3
	TXD
	BA
	103
	Transmit Data

	4
	DTR
	CD
	108.2
	Data Terminal Ready

	5
	GND
	AB
	102
	System Ground

	6
	DSR
	CC
	107
	Data Set Ready

	7
	RTS
	CA
	105
	Request to Send

	8
	CTS
	CB
	106
	Clear to Send

	9
	RI
	CE
	125
	Ring Indicator


	 
	 
	 

	 
	Serial (PC 25)
	



	 
	[image: image14.png]P

T




	25 PIN D-SUB MALE at the computer.


	Pin
	Name
	Description

	1
	SHIELD
	Shield Ground

	2
	TXD
	Transmit Data

	3
	RXD
	Receive Data

	4
	RTS
	Request to Send

	5
	CTS
	Clear to Send

	6
	DSR
	Data Set Ready

	7
	GND
	System Ground

	8
	CD
	Carrier Detect

	9
	n/c
	 

	10
	n/c
	 

	11
	n/c
	 

	12
	n/c
	 

	13
	n/c
	 

	14
	n/c
	 

	15
	n/c
	 

	16
	n/c
	 

	17
	n/c
	 

	18
	n/c
	 

	19
	n/c
	 

	20
	DTR
	Data Terminal Ready

	21
	n/c
	 

	22
	RI
	Ring Indicator

	23
	n/c
	 

	24
	n/c
	 

	25
	n/c
	 


	 
	 
	 

	 
	Parallel (PC)
	



	 
	 [image: image16.png]13

T4

%




	25 PIN D-SUB FEMALE at the PC.


	Pin
	Name
	Description

	1
	/STROBE
	Strobe

	2
	D0
	Data Bit 0

	3
	D1
	Data Bit 1

	4
	D2
	Data Bit 2

	5
	D3
	Data Bit 3

	6
	D4
	Data Bit 4

	7
	D5
	Data Bit 5

	8
	D6
	Data Bit 6

	9
	D7
	Data Bit 7

	10
	/ACK
	Acknowledge

	11
	BUSY
	Busy

	12
	PE
	Paper End

	13
	SEL
	Select

	14
	/AUTOFD
	Autofeed

	15
	/ERROR
	Error

	16
	/INIT
	Initialize

	17
	/SELIN
	Select In

	18
	GND
	Signal Ground

	19
	GND
	Signal Ground

	20
	GND
	Signal Ground

	21
	GND
	Signal Ground

	22
	GND
	Signal Ground

	23
	GND
	Signal Ground

	24
	GND
	Signal Ground

	25
	GND
	Signal Ground


	 
	ATA Internal
	ATA=AT bus Attachment.

	 
	[image: image17.png]



	40 PIN IDC MALE at the controller & peripherals.

	 
	[image: image18.png]40

£





	40 PIN IDC FEMALE at the cable.


	Pin
	Name
	Description

	1
	/RESET
	Reset

	2
	GND
	Ground

	3
	DD7
	Data 7

	4
	DD8
	Data 8

	5
	DD6
	Data 6

	6
	DD9
	Data 9

	7
	DD5
	Data 5

	8
	DD10
	Data 10

	9
	DD4
	Data 4

	10
	DD11
	Data 11

	11
	DD3
	Data 3

	12
	DD12
	Data 12

	13
	DD2
	Data 2

	14
	DD13
	Data 13

	15
	DD1
	Data 1

	16
	DD14
	Data 14

	17
	DD0
	Data 0

	18
	DD15
	Data 15

	19
	GND
	Ground

	20
	KEY
	Key (Pin missing)

	21
	DMARQ
	DMA Request

	22
	GND
	Ground

	23
	/DIOW
	Write Strobe

	24
	GND
	Ground

	25
	/DIOR
	Read Strobe

	26
	GND
	Ground

	27
	IORDY
	I/O Ready

	28
	SPSYNC:CSEL
	Spindle Sync or Cable Select

	29
	/DMACK
	DMA Acknowledge

	30
	GND
	Ground

	31
	INTRQ
	Interrupt Request

	32
	/IOCS16
	IO ChipSelect 16

	33
	DA1
	Address 1

	34
	PDIAG
	Passed Diagnostics

	35
	DA0
	Address 0

	36
	DA2
	Address 2

	37
	/IDE_CS0
	(1F0-1F7)

	38
	/IDE_CS1
	(3F6-3F7)

	39
	/ACTIVE
	Led driver

	40
	GND
	Ground


	 
	 
	 

	 
	RJ-45              


	 

	 
	[image: image20.jpg][Straight Through Cable|
| RJ45PIN | RJ45 PIN

| crossover Cable
|RJ45 PIN RJ45 PIN





	  

 

 


	 
	S-Video
	



	 
	[image: image22.png]



	4 PIN MINI-DIN FEMALE at the peripheral.


	Pin
	Name
	Description

	1
	GND
	Ground (Y)

	2
	GND
	Ground (C)

	3
	Y
	Intensity (Luminance)

	4
	C
	Color (Chrominance)


	 
	 
	 

	 
	Sony Playstation Port
	



	 
	  1  2  3   4  5  6   7  8  9
 -------------------------------
 | o  o  o | o  o  o | o  o  o |  
 \_____________________________/
 
	9 PIN CONNECTOR


	Pin
	Name
	Description

	1
	DATA
	Data

	2
	CMD
	Command

	3
	N/C (9 V unused)
	Not connected

	4
	GND
	Ground

	5
	VCC
	Vcc

	6
	ATT
	ATT select

	7
	CLK
	Clock

	8
	N/C
	Not connected

	9
	ACK
	Acknowledge


	 
	 
	



	 
	TABELA AWG
	AWG=American Wire Gauge standard


	Gauge
	Diam
	Area
	R
	I at 3A/mm2

	AWG
	mm
	mm2
	ohm/km
	mA

	46
	0,04
	0,0013
	13700
	3,8

	44
	0,05
	0,0020
	8750
	6

	42
	0,06
	0,0028
	6070
	9

	41
	0,07
	0,0039
	4460
	12

	40
	0,08
	0,0050
	3420
	15

	39
	0,09
	0,0064
	2700
	19

	38
	0,10
	0,0078
	2190
	24

	37
	0,11
	0,0095
	1810
	28

	 
	0,12
	0,011
	1520
	33

	36
	0,13
	0,013
	1300
	40

	35
	0,14
	0,015
	1120
	45

	 
	0,15
	0,018
	970
	54

	34
	0,16
	0,020
	844
	60

	 
	0,17
	0,023
	757
	68

	33
	0,18
	0,026
	676
	75

	 
	0,19
	0,028
	605
	85

	32
	0,20
	0,031
	547
	93

	30
	0,25
	0,049
	351
	147

	29
	0,30
	0,071
	243
	212

	27
	0,35
	0,096
	178
	288

	26
	0,40
	0,13
	137
	378

	25
	0,45
	0,16
	108
	477

	24
	0,50
	0,20
	87,5
	588

	 
	0,55
	0,24
	72,3
	715

	 
	0,60
	0,28
	60,7
	850

	22
	0,65
	0,33
	51,7
	1,0 A

	 
	0,70
	0,39
	44,6
	1,16 A

	 
	0,75
	0,44
	38,9
	1,32 A

	20
	0,80
	0,50
	34,1
	1,51 A

	 
	0,85
	0,57
	30,2
	1,70 A

	19
	0,90
	0,64
	26,9
	1,91 A

	 
	0,95
	0,71
	24,3
	2,12 A

	18
	1,00
	0,78
	21,9
	2,36 A

	 
	1,10
	0,95
	18,1
	2,85 A

	 
	1,20
	1,1
	15,2
	3,38 A

	16
	1,30
	1,3
	13,0
	3,97 A

	 
	1,40
	1,5
	11,2
	4,60 A

	 
	1,50
	1,8
	9,70
	5,30 A

	14
	1,60
	2,0
	8,54
	6,0 A

	 
	1,70
	2,3
	7,57
	6,7 A

	13
	1,80
	2,6
	6,76
	7,6 A

	 
	1,90
	2,8
	6,05
	8,5 A

	12
	2,00
	3,1
	5,47
	9,4 A


